- Explain the functions of ALE and IO/M signals of the 8085 microprocessor.

- Draw the timing diagram for RST instruction.

- The instruction LDA 2050H copies the contents of the memory location 2050H into the accumulator. It is a 3-byte instruction with 4 machine cycles and 13 T-states. Illustrate the fourth machine cycle, clearly indicating the control signals, contents of address bus and data bus.

- Specify the register contents and flag status for the assembly language program:

MVI A, 87H

MVI B, 79H

ADD B

HLT

- Consider the following assembly language program:

MVI A, 8FH

MVI B, 68H

SUB B

ANI 0FH

STA 2070H

HLT

What is the final value at 2070H? Give your answer in hexadecimal.

- Explain the need to demultiplex the bus AD7-AD0.

- Discuss with example the various addressing modes used in 8085?

- LXI rp, d16 is a 3-byte instruction, which is used to load 16-bit address into the register pair rp. Eg: if the instruction is LXI D, FE50H. It means that the FE50H is loaded into the DE register pair. Draw timing diagram for LXI D, FE50H.

- Mention the purpose of SID and SOD lines in 8085 microprocessors.

- Consider the following assembly language program:

MVI B, 4FH

MVI C, 78H

MOV A, C

ADD B

OUT 07H

HLT

Calculate the program size (in bytes), M-cycles, and T-states. Calculate the time required to execute this entire program, if the clock frequency is 2 MHz.