- Consider a non-pipelined processor with a clock rate of 2.5 gigahertz and average cycles per instruction of four. The same processor is upgraded to a pipelined processor with five stages; but due to the internal pipeline delay, the clock speed is reduced to 2 gigahertz. Assume that there are no stalls in the pipeline. Calculate the speedup.

- The stage delays in a 4-stage pipeline are 800, 500, 400 and 300 picoseconds. The first stage (with delay 800 picoseconds) is replaced with a functionally equivalent design involving two stages with respective delays 600 and 350 picoseconds. Calculate the increase in throughput (in percent).

- To execute an instruction by a 32-bit machine the following steps are carried out: Fetch, Decode, Execution, Memory access and Store, each of which takes 1 clock period. In a pipelined execution of a 5-step task, a new instruction is read and it takes 1 ns. If there are 100 instructions in sequence, then calculate the speedup ratio of pipe line processing system over an equivalent non pipeline processing system.

- A non-pipelined single cycle processor operating at 100 MHz is converted into a synchronous pipelined processor with five stages requiring 2.5 ns, 1.5 ns, 2 ns, 1.5 ns and 2.5 ns respectively. The delay of the latches is 0.5 ns. Calculate the speedup.

- The 5-stages of a processor have the following latencies: 300 ns, 400 ns, 350 ns, 550 ns and 100 ns.

- For a non-pipelined processor, calculate: cycle time, throughput.

- For pipelined processor, calculate: cycle time, throughput. Assume that each pipeline stage costs 20 ns extra for the buffer registers.

- If any one of the pipeline stages could be split into two halves, which one would you choose and why. Calculate the new cycle time, and throughput.

- Consider a non-pipelined processor. Assume that it has a 1 ns clock cycle and that it uses 4 cycles for ALU operations and branches and 5 cycles for memory operations. Assume that the relative frequencies of these operations are 40%, 20% and 40% respectively. Suppose that due to overheads, pipelining the processor adds 0.2ns of overhead. Ignore any pipeline hazard, calculate how much speedup will we gain from a pipeline.