Q.1	 Consider a network with 6 routers R1 to R6 connected with links having weights as shown in the following diagram

All the routers use the distance vector based routing algorithm to update their routing tables. Each router starts with its routing table initialized to contain an entry for each neighbour with the weight of the respective connecting link. After all the routing tables stabilize, how many links in the network will never be used for carrying any data?

Answer :- 	2

We can check one by one all shortest distances. When we check for all shortest distances for Ri we don’t need to check its distances to R0 to Ri-1 because the network graph is undirected.
Following will be distance vectors of all nodes.

Shortest Distances from R1 to R2, R3, R4, R5 and R6
R1 (5, 3, 12, 12, 16)
Links used: R1-R3, R3-R2, R2-R4, R3-R5, R5-R6

Shortest Distances from R2 to R3, R4, R5 and R6
R2 (2, 7, 8, 12)
Links used: R2-R3, R2-R4, R4-R5, R5-R6

Shortest Distances from R3 to R4, R5 and R6
R3 (9, 9, 13)
Links used: R3-R2, R2-R4, R3-R5, R5-R6

Shortest Distances from R4 to R5 and R6
R4 (1, 5)
Links used: R4-R5, R5-R6

Shortest Distance from R5 to R6
R5 (4)
Links Used: R5-R6

If we mark, all the used links one by one, we can see that following links are never used.
R1-R2
R4-R6

b) Suppose the weights of all unused links in the previous question are changed to 2 and the distance vector algorithm is used again until all routing tables stabilize. How many links will now remain unused?
(A) 0
(B) 1
(C) 2
(D) 3

Answer :- 	1
After the weights of unused links () are changed to following graph.

Following will be distance vectors of all nodes
R1 (2, 3, 9, 10, 11)
Links used: R1-R2, R1-R3, R2-R4, R4-R5, R4-R6

R2 (2, 7, 8, 9)
Links used: R2-R3, R2-R4, R4-R5, R4-R6

R3 (9, 9, 11)
Links used: R3-R2, R2-R4, R3-R5, R4-R6

R4 (1, 2)
Links used: R4-R5, R4-R6

R5 (3)
Links Used: R5-R4, R4-R6

If we mark, all the used links one by one, we can see that all links are used except the following link.
R5-R6

Q.2	Consider an IP packet with a length of 4,500 bytes that includes a 20-byte IPv4 header ans 40-byte TCP header. The packet is forwarded to an IPv4 router that supports a Maximum Transmission Unit (MTU) of 600 bytes. Assume that the length of the IP header in all the outgoing fragments of this packet is 20 bytes. Assume that the fragmentation offset value stored in the first fragment is 0.

The fragmentation offset value stored in the third fragment is ______ .

Answer: 	144

Explanation: MTU = 600 bytes and IP Header = 20 bytes
So, Payload will be 600 – 20 = 580 bytes

580 is not multiple of 8, but we know fragment size should be multiple of 8. So fragment size = 576 bytes

Kth fragmentation offset value = Fragment Size * (Kth fragment – 1) / Scaling Factor
Offset value of 3rd fragment = 576 * (3 – 1) / 8 = 144

Q.3	An IP router with a Maximum Transmission Unit (MTU) of 1500 bytes has received an IP packet of size 4404 bytes with an IP header of length 20 bytes. The values of the relevant fields in the header of the third IP fragment generated by the router for this packet are

(A) MF bit: 0, Datagram Length: 1444; Offset: 370
(B) MF bit: 1, Datagram Length: 1424; Offset: 185
(C) MF bit: 1, Datagram Length: 1500; Offset: 37
(D) MF bit: 0, Datagram Length: 1424; Offset: 2960


Answer: (A)

Explanation:

Number of packet fragments = ⌈ (total size of packet)/(MTU) ⌉

= ⌈ 4404/1500 ⌉

= ⌈ 2.936 ⌉

= 3

So Datagram with data 4404 byte fragmented into 3 fragments.

The first frame carries bytes 0 to 1479 (because MTU is 1500 bytes and HLEN is 20 byte so the total bytes in fragments is maximum 1500-20=1480). the offset for this datagram is 0/8 = 0.

The second fragment carries byte 1480 to 2959. The offset for this datagram is 1480/8 = 185.finally the third fragment carries byte 2960 to 4404.the offset is 370.and for all fragments except last one the M bit is 1.so in the third bit M is 0..

Q.4	In an IPv4 datagram, the M bit is 0, the value of HLEN is 10, the value of total length is 400 and the fragment offset value is 300. The position of the datagram, the sequence numbers of the first and the last bytes of the payload, respectively are


Answer: 2400 and 2759

Explanation: M = 0 indicates that this packet is the last packet among all fragments of original packet. So the answer is either A or C.

It is given that HLEN field is 10. Header length is number of 32 bit words. So header length = 10 * 4 = 40
Also, given that total length = 400.
Total length indicates total length of the packet including header.
So, packet length excluding header = 400 – 40 = 360
Last byte address = 2400 + 360 – 1 = 2759 (Because numbering starts from 0)

Q.5	An IP datagram of size 1000 bytes arrives at a router. The router has to forward this packet on a link whose MTU (maximum transmission unit) is 100 bytes. Assume that the size of the IP header is 20 bytes. The number of fragments that the IP datagram will be divided into for transmission is :
 

Answer: 	13

Explanation: MTU = 100 bytes
Size of IP header = 20 bytes
So, size of data that can be transmitted in one fragment = 100 – 20 = 80 bytes
Size of data to be transmitted = Size of datagram – size of header = 1000 – 20 = 980 bytes
 
Now, we have a datagram of size 1000 bytes.
So, we need ceil(980/80) = 13 fragments.
 
Thus, there will be 13 fragments of the datagram.
So, D is the correct choice.

Q.6	Host A sends a UDP datagram containing 8880 bytes of user data to host B over an Ethernet LAN. Ethernet frames may carry data up to 1500 bytes (i.e. MTU = 1500 bytes). Size of UDP header is 8 bytes and size of IP header is 20 bytes. There is no option field in IP header. How may total number of IP fragments will be transmitted and what will be the contents of offset field in the last fragment?

Answer: 	7 and 1110

Explanation:

UDP data = 8880 bytes

UDP header = 8 bytes

IP Header = 20 bytes

Total Size excluding IP Header = 8888 bytes.

Number of fragments  = ⌈ 8888 / 1480 ⌉

= 7

Refer the Kurose book slides on IP (Offset is always scaled by 8)

Offset of last segment = (1480 * 6) / 8 = 1110

Q.7	An IP packet has arrived with the first 8 bits as shown: Í 01000010 The receiver discards the packet. Why?

Solution:  There is an error in this packet. The 4 left-most bits (0100) show the version, which is correct. The next 4 bits (0010) show the header length, which means (2 × 4 = 8), which is wrong. The minimum number of bytes in the header must be 20. The packet has been corrupted in transmission.

Q.8	In an IP packet, the value of HLEN is 1000 in binary. How many bytes of options are being carried by this packet?

Solution: The HLEN value is 8, which means the total number of bytes in the header is 8 × 4 or 32 bytes. The first 20 bytes are the main header, the next 12 bytes are the options.

Q.9	In an IP packet, the value of HLEN is 5base16 and the value of the total length field is 0028base16. How many bytes of data are being carried by this packet?

Solution: The HLEN value is 5, which means the total number of bytes in the header is 5 × 4 or 20 bytes (no options). The total length is 40 bytes, which means the packet is carrying 20 bytes of data (40 −20).

Q.10

Q.11	A packet has arrived in which the offset value is 100, the value of HLEN is 5 and the value of the total length field is 100. What is the number of the first byte and the last byte?

Solution: The first byte number is 100 × 8 = 800. The total length is 100 bytes and the header length is 20 bytes (5 × 4), which means that there are 80 bytes in this datagram. If the first byte number is 800, the last byte number must 879.