# Experiment 7	Digital Electronics	Monsoon 2018

Latch, Flip-flop and Ripple Counter

In this experiment, you will study the two basic modules required for the design of sequential circuits – LATCH for asynchronous circuits, and FLIP-FLOP for synchronous circuits. This will be followed by a study of simple Ripple Counters using flip-flops. The chips you will be using in this experiment are as follows:

- CD/HEF 4001 NOR gate, (ii) 74HC00 NAND gate, (iii) CD 4013 D Flip-flop and (iii) CD 4027 JK Flip-flop.

- Latches

  - Assemble a NOR latch as given in Fig. 7.1, using two gates in the CD/HEF 4001 chip, with the two outputs Q and Q’ connected to two LED displays. Observe and tabulate the sequence of Q and Q’ in response to the following sequence of the two inputs S (Set : Q ← 1) and R (cleaR : Q ← 0) applied by connecting the two inputs to two Input Switches:	S R = 01, 00, 10, 00, 01, 11, 00, 10, 11, 00. 	Interpret the observed result and verify the latch operation.

D

En’

Q

Q’

R

S

Q

Q’

### Fig. 7.1 NOR Latch

### Fig. 7.2 D Latch using NOR gates

  - Disconnect the Input Switches from the S and R inputs and add two NOR gates from the same 4001 chip and another NAND gate configured as an inverter from a 74HC00 chip to obtain a D latch as given in Fig. 7.2. Apply the two inputs D and En’ from two Input Switches, and tabulate the outputs Q and Q’ for all four combinations of the two inputs. Verify that Q = D if En’ = 0, and Q is independent of D  if En’ = 1.

1        2        3         4       5        6         7       8     888

16      15      14      13       12      11      10       9

VDD     Q        Q’     CK      RD       K       J       SD

CD4027             FF2

Dual J-K FF        FF1

Q       Q’     CK      RD          K         J       SD   Gnd

VDD     Q        Q’      CK      RD         D      SD

CD4013           FF2

Dual D FF        FF1

Q        Q’     CK      RD      D       SD   Gnd

1         2        3        4        5        6        7

14      13       12     11       10       9        8

Flip-flops

Table 7.1 Truth Tables of Flip-flops

| SD | RD | Qnext |
| --- | --- | --- |
| 0 | 0 | Qold |
| 1 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 1 | ? |

| J | K | Qnext |
| --- | --- | --- |
| 0 | 0 | Qold |
| 1 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 1 | Qold’ |

| D | Qnext |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

- SD-RDTruth Table	(b) D Truth Table	(c) J-K Truth table

The pin connections of the CD4013 and the CD4027 chips are given above. Note that each FF in the chip has a direct (asynchronous) Set input SD and a direct (asynchronous) clear input RD, besides the synchronous inputs D / JK. This enables one either to Set (Q ← 1) or cleaR (Q ← 0) the flip-flop to set up initial conditions as desired, by making SD RD = 1 0 or SD RD = 0 1 respectively. SD RD are asynchronous inputs as in the case of a latch, and hence SD RD = 1 1 is an illegal input combination, which forces both Q ← 1 and Q’ ← 1.

Note that the truth table of a D FF is the same as that of a D latch with En’ = 0. In both the 4013 D FF and the 4027 JK FF, the FFs accept fresh data when CK = 0, and hold the data when CK = 1. Thus the effect of the CK input is exactly like that of the En’ input of latches, with the difference that while Enable is an asynchronous control signal asserted whenever the latch is required to accept fresh data, Clock is generally a running sequence of pulses applied to all the FFs that are required to behave in synchronization with one another. The En’ input is thus treated as a LEVEL-sensitive input, and the CK input, as an EDGE-sensitive input.

  - The Digital Lab Kit has a Clock Generator, with a choice of a Manual Clock applied through a Switch and a Free-running Clock having two options: Slow Clock (time-period  2s) clock and Fast Clock (time-period  2ms). Set the Clock in the Manual mode, and verify that clock pulses are generated  by moving the Manual Clock Switch to and fro, as indicated by the Clock indicator LEDs switching between Red and Green.

  - Test each type of FF one by one, by connecting the SD RD and D / J-K inputs of the FF to Input Switches, the CK input to the CLK OUT pin, and the Q and Q’ outputs to LED displays. Verify the SD RD Truth Table given in Table 7.1(a), by applying the sequence SD RD = 01, 00, 10, 00, 01, 11, 10, 00, 11, 01, 00, 11, 00, with the D / J-K and CK inputs at any fixed level, and tabulating the sequence of Q and Q’.

- Verify that the action of SD and RD is unaffected by the levels of the synchronous and clock inputs by repeating step B.1 for each type of FF with a few different combinations of D / J-K and CK. However, leaving the synchronous inputs open may lead to malfunctioning because of noise pickup.

- With SD = RD = 0, verify the Truth Table given Table 7.1(b) for the D FF, by observing Q and Q’ before and after applying a Manual Clock  pulse (0-1-0) with D = 0 and with D = 1. Verify that the flip-flop is positive edge-triggered, i.e. the LOW HIGH clock edge causes the transition in the output.

- In a similar fashion, verify the J-K Truth Table given in Table 7.1(c) for the JK FF, applying synchronous inputs J K in the following sequence and applying a Manual Clock pulse for every combination of J K:

J K = 01, 00, 10, 00, 01, 10, 01, 11, 00, 11, 00.

- Binary Ripple Counter

A binary counter is required to count the number of applied pulses in the binary number sequence – either in the increasing order (UP counting) or in the decreasing order (DOWN counting). A ripple counter consists of T flip-flops with T = 1 connected in cascade. As T flip-flops are not normally available as a commercial product, a T FF is obtained either from a JK flip-flop by making J = K = 1, or from a D flip-flop by making D = Q’.

- Change the setting of the Clock Generator from Manual to Slow Free-running, and verify this mode of the Clock output by observing the Clock indicator LED alternating between the ON and OFF states.

- Mark all the pin numbers of the two chips in the circuit diagram of the binary DOWN counter using two JK flip-flops and two D flip-flops, given in Fig. 7.3(a), and assemble the complete circuit on the breadboard. Connect all the SD inputs together to one Input Switch, all the RD inputs together to another Input Switch, and the four FF outputs to four LED Displays.

- Apply SD RD = 0 1 to initialise the counter to the starting state 0000 for UP counting.

- Now enable counting by making SD = RD = 0. Observe and tabulate the state sequence (Q3 Q2 Q1 Q0) under this condition.

SD

Q0                  Q1             Q2              Q3

1    J   Q     1  J   Q         D   Q         D   Q

Clock

1    K  Q’    1  K  Q’              Q’             Q’

RD

SD

Q0                  Q1             Q2              Q3

1    J   Q     1  J   Q         D   Q         D   Q

Clock

1    K  Q’    1  K  Q’              Q’             Q’

RD

Repeat steps C.2, C.3 and C.4 for the binary DOWN counter shown in Fig. 7.3(b), with the connections for SD RD unchanged, initialising the counter to 1111 for DOWN counting by applying SD RD = 1 0.

Fig. 6.3(a) Binary UP Counter	Fig. 6.3(b) Binary DOWN Counter